SYMBIOSIS INSTITUTE OF TECHNOLOGY, NAGPUR

SECOND SEMESTER SESSION 2021-2022

SUBJECT :- ENGINEERING GRAPHICS

SHEET NO 1: PROJECTION OF LINES

DATE OF SUBMISSION 31/05/2022

- The projectors of ends of a line AB are 60 mm apart. End A is 50 mm above the HP and 15 mm in front and VP. While the end B is 10 mm above HP and 60 mm in front of VP. Draw its projections and determine its inclination with HP and VP.

- A line AB 75 mm long is inclined at 40° to the VP. Its top view measures 60 mm. One end of the line is 15 mm above HP and 25 mm infront of VP. Draw its projections and determine its inclination with HP.

- The top view and the front view of the line CD measure 75mm and 63mm respectively. The line is inclined to H.P. and V.P. by 40º and 45º respectively. The end C is 5mm above the HP and 20mm in front of VP. Draw the projections of the line CD and find True length.

- A line AB has its end A 15mm above HP and 10mm in front of VP. The end B is 60mm above HP. The distance between the end projectors is 50mm. The line is inclined to HP by 25º. Draw the projections and find its in inclination with VP and the true length of line AB.

- Line AB, 80mm long is inclined at 35º to HP, and 55º to V.P. If point A is 20mm away from both the planes, the Draw the projections of line AB and determine the value of its apparent angles.

- A line AB measures 70mm and is inclined at 30º to HP. The front view of the line measures 50mm. Draw the projections if point A is 15mm above H.P. and 10mm in front of VP.

SYMBIOSIS INSTITUTE OF TECHNOLOGY, NAGPUR

SECOND SEMESTER SESSION 2021-2022

SUBJECT :- ENGINEERING GRAPHICS

SHEET NO 2: PROJECTION OF PLANES

DATE OF SUBMISSION 05/06/2022

- The one side of a regular hexagon of 30 mm side is in HP, such that surface is inclined at 45°to HP and the side is  inclined at 30° to the VP. Draw the projections of plane.

- A regular pentagon of 35mm side is resting on one of its side on H.P and the corner opposite to that edge is 30mm above H.P. and the side on which it is resting makes an angle 45º with V.P. Draw its projections (three views)

- A regular Hexagon of 30mm side is resting on one of its corner in VP, such that its surface is inclined at 35º to VP. Draw the projections if the front view of the diagonal from the resting corner makes an angle of 45º with HP.

- A thin plate inclined at 45º to HP appears as a regular pentagon of 30 mm side in T.V. considering that the plate is resting on one of its side in HP, draw the projections and determine the true shape of the plate.

SYMBIOSIS INSTITUTE OF TECHNOLOGY, NAGPUR

SECOND SEMESTER SESSION 2021-2022

SUBJECT :- ENGINEERING GRAPHICS

SHEET NO 3: PROJECTION OF SOLIDS

DATE OF SUBMISSION 10/06/2022

- A cone of base diameter 50 mm and axis 70 mm is resting on a point on circumference of base on VP such that its base is inclined at 60° to VP. Draw the projections FV of axis is inclined at 45° to HP.

- Draw the projection of a cube of 40 mm long edges resting on the HP on one of its corner with a solid diagonal perpendicular to the VP.

- A pentagonal prism, with side of base 40mm and axis 70mm is resting on a corner of its base on the HP with a longer edge containing that corner is inclined at 45º to H.P. If the Top view of the axis is inclined at 45º to V.P. Draw the projections.

- A hexagonal pyramid, side of base 30 mm and height 70mm lies on one of its triangular faces on the H. P. The edge of base lying on the H. P. makes an angle of 45º with the V. P.